Chapter 4D: BLE Centrals

Time 2 ½ Hours

This chapter introduces you to the Central side of the BLE connection.  By the end of this chapter you should be able to create a BLE Central that finds the right BLE Peripheral, connects to it, Reads, Writes and Enables/Handles notifications.  It should also be able to perform the GATT Service Discovery Procedure to find the Handles of the Services, Characteristics and Descriptors on the GATT Server.

4D.1	GAP Roles, the Observer and the Central	2

4D.2	Scanning	2

4D.3	Connecting	5

4D.4	Attribute Protocol & More GATT Procedures	6

4D.4.1 GATT Client Read	7

4D.4.2 GATT Client Write and Write Command	9

4D.4.3 GATT Client Notify and Indicate	10

4D.4.4 GATT Group	11

4D.4.5 GATT Client Read by Group Type	11

4D.4.6 GATT Client Find by Type Value	12

4D.4.7 GATT Client Read by Type	12

4D.4.8 GATT Client Find Information	13

4D.5	GATT Procedure: Service Discovery	13

4D.5.1 Service Discovery Algorithm	13

4D.5.2 WICED Service Discovery	14

4D.6	Running a GATT Server	15

4D.7	Exercises	16

Exercise - 4D.1 Make an Observer	16

Exercise - 4D.2 Add a filter to show only your device	16

Exercise - 4D.3 Update to connect to your Peripheral device & turn on/off the LED	16

Exercise - 4D.4 (Advanced) Add keys to turn on CCCD	18

Exercise - 4D.5 (Advanced) Make your project do Service Discovery	18

Exercise - 4D.6 (Advanced) Run the advertising scanner	21

# GAP Roles, the Observer and the Central

In the previous three chapters the focus has been on BLE Peripherals.  Instead of dividing the world into Peripheral and Central, it would have been more technically correct to say that Bluetooth Low Energy has four GAP device roles:

- Broadcaster:	A device that only advertises

- Peripheral:	A device that can advertise and be connected to

- Observer:	A device that passively listens to advertisers

- Central: 	A device that can listen to advertisers and create a connection to a Peripheral

So, the previous chapters were really focused on Broadcasters and Peripherals.  But what about the other side of the connection?  The answer to that question is the focus of this chapter.

# Scanning

In the previous chapters I talked about how you create different Peripherals that Advertise their existence and some data.  How does a Central find this information?  And how does it use that information to get connected?

First, you must put the WICED BLE device into scanning mode.  You do this with a simple call to wiced_ble_bt_scan.  This function takes three arguments.

- The first argument is a wiced_bt_ble_scan_type_t which tells the controller to either turn off, scan fast (high duty) or scan slowly (low duty).

enum wiced_bt_ble_scan_type_e

{

BTM_BLE_SCAN_TYPE_NONE,         /**< Stop scanning */

BTM_BLE_SCAN_TYPE_HIGH_DUTY,    /**< High duty cycle scan */

BTM_BLE_SCAN_TYPE_LOW_DUTY      /**< Low duty cycle scan */

};

The actual parameters of scan fast/slow are configured in the wiced_bt_cfg_settings_t structure typically found in wiced_bt_cfg.c.

- The next argument is a wiced_bool_t that tells the scanner to filter or not. If you enable the filter, the scanner will only call you back one time for each unique BD ADDR that it hears even if the advertising packet changes.

- The final argument is a function pointer to a callback function that looks like this:

void myScanCallback(wiced_bt_ble_scan_results_t *p_scan_result, uint8_t *p_adv_data);

Each time that your Central hears an advertisement, it will call your callback with a pointer to a scan result structure with information about the device it just heard, and a pointer to the raw advertising data. The scan result structure simply has the Bluetooth Device Address, the address type, what type of advertisement packet, the RSSI and a flag like this:

/** LE inquiry result type */

typedef struct

{

wiced_bt_device_address_t       remote_bd_addr;        /**< Device address */

uint8_t                         ble_addr_type;         /**< LE Address type */

wiced_bt_dev_ble_evt_type_t     ble_evt_type;          /**< Scan result event type */

int8_t                          rssi;

uint8_t                         flag;

} wiced_bt_ble_scan_results_t;

In your advertising callback function you can then parse the advertising data to decide what do next.  WICED provides you a function called wiced_bt_ble_check_advertising data which can help you find information in the packet.  Recall that every advertising packet is broken up into fields that each have a type.  The wiced_bt_ble_check_advertising function will search the advertising packet looking for a field that you specify and then, if it finds that field, will return a pointer to the field and the length (via a pointer). For example, you might have this inside the callback function to search for a Service UUID:

uint8_t len;

uint8_t *findServiceUUID = wiced_bt_ble_check_advertising_data(p_adv_data, BTM_BLE_ADVERT_TYPE_128SRV_COMPLETE, &len);

After making this call, findServiceUUID will either be 0 (it didn’t find the field) or will be a pointer to the bytes that make up the Service UUID.  In addition, len with either be 0 or it will be the number of bytes in that field.   Remember that the enumeration wiced_bt_ble_advert_type_e in the file wiced_bt_ble.h has a list of the legal advertising field types.

So, now what?  Well consider the two cases in the screenshot from the Advertising Scanner (that project is Exercise - 4D.6  in this chapter).  The results shown in the screenshot are from the devices created in Chapter 4B Exercise 03 and Chapter 4C Exercise 01. There are two different devices advertising, one named “key_pair” and one named “key manu”.  You can see the raw bytes of the advertising packet and the decode of those bytes.

If you were looking for a device named key_pair you could do something like this:

uint8_t *name = wiced_bt_ble_check_advertising_data(p_adv_data,

BTM_BLE_ADVERT_TYPE_NAME_COMPLETE, &length);

if(name && strncmp(name,"key_pair",length))

{

WICED_BT_TRACE("Host = %B keypair\r\n ", p_scan_result->remote_bd_addr);

}

If you were looking for a device that was advertising the keypair Service UUID you might do this (notice that the UUIDs are stored little endian in the advertising packet):

static const uint8_t serviceUUID[]={0x31,0x01,0x9B,0x5F,0x80,0x00,0x00,0x80,0x00,0x10,0x00,0x00,0xB5,0xCA,0x03,0x00};

uint8_t *findServiceUUID = wiced_bt_ble_check_advertising_data(p_adv_data,

BTM_BLE_ADVERT_TYPE_128SRV_COMPLETE,&len);

if(findServiceUUID && memcmp(serviceUUID,serviceUUID,16) == 0)

{

WICED_BT_TRACE("Host = %B Found Service UUID\r\n ", p_scan_result->remote_bd_addr);

}

If you were looking for the Peripheral that was advertising the manufacturer's data with the Cypress manufacturer ID of 0x0131 and 0xFA for the data (which is what “key manu” is advertising) you might do this (again, notice the manufacturer ID is little endian in the advertising packet):

uint8_t *mfgData = wiced_bt_ble_check_advertising_data(p_adv_data,

BTM_BLE_ADVERT_TYPE_MANUFACTURER, &length);

if(mfgData && mfgData[0] == 0x31 && mfgData[1] == 01 && mfgData[2] == 0xFA)

{

WICED_BT_TRACE("Host = %B length=%d ", p_scan_result->remote_bd_addr, length);

}

There are several functions which can be useful in comparing data for when you want to identify a particular device such as:

- Strcmp(char *srt1, char *str2) and strncmp(char *str1, char *str2, int n) which compare two strings. Both functions take two pointers to strings.  The strncmp function also takes an int which is the maximum number of characters to compare (this is safer).  Both functions return a 0 if the strings MATCH (the opposite of a normal function).

- memcmp(unit8_t *p1,uint8_t *p2, int size) allows you to compare two blocks of memory.  It returns 0 if the two blocks of memory are the same.

- If you have two variables of type wiced_uuid_t you can compare them with the wiced_bt_util_uuid_cmp function. Like the functions above, it returns 0 if the two UUIDs match.

int wiced_bt_util_uuid_cmp(wiced_bt_uuid_t *p_uuid1, wiced_bt_uuid_t *p_uuid2);

# Connecting

Now that you have found a device that you are interested in what next?  To make a connection you just call wiced_bt_gatt_le_connect:

wiced_bool_t wiced_bt_gatt_le_connect(wiced_bt_device_address_t bd_addr,

wiced_bt_ble_address_type_t bd_addr_type,

wiced_bt_ble_conn_mode_t conn_mode,

wiced_bool_t is_direct);

The bd_addr and bd_addr_type are passed in to the callback as part of the scan result. The conn_mode is an enumeration with three possible values which determines how fast the connection is established (and how much power is consumed to establish the connection):

enum wiced_bt_ble_conn_mode_e

{

BLE_CONN_MODE_OFF,                  /**< Stop initiating */

BLE_CONN_MODE_LOW_DUTY,             /**< slow connection scan parameter */

BLE_CONN_MODE_HIGH_DUTY             /**< fast connection scan parameter */

};

The final argument should be set to WICED_TRUE. That feature is going to be deprecated.

When the connection has been made, the GATT callback that you registered with wiced_bt_gatt_register will be called with the event GATT_CONNECTION_STATUS_EVT.  The parameter passed to you will be of type wiced_bt_gatt_connection_status_t which contains a bunch of information about the connection.

typedef struct

{

uint8_t                         *bd_addr;   /**< Remote device address */

wiced_bt_ble_address_type_t     addr_type;  /**< Remmote device address type */

uint16_t                        conn_id;    /**< ID of the connection */

wiced_bool_t                    connected;  /**< TRUE/FALSE connected/disconnected */

wiced_bt_gatt_disconn_reason_t  reason;     /**< Reason code (see @link

wiced_bt_gatt_disconn_reason_e

wiced_bt_gatt_disconn_reason_t @endlink) */

wiced_bt_transport_t            transport;  /**< Transport type of the connection */

uint8_t                         link_role;  /**< Link role on this connection */

} wiced_bt_gatt_connection_status_t;

Typically, you would save the conn_id so that you can perform Reads and Writes to the Peripheral.  If you were going to support multiple connections you might make a table of connection ID / Bluetooth Address tuples.

# Attribute Protocol & More GATT Procedures

In the previous chapters I introduced you to the Peripheral side of several GATT Procedures. Specifically, Read, Write and Notify.  Moreover, in those chapters you learned how to create WICED firmware to respond to those requests.  You will recall that each of those GATT Procedures are mapped into one or more Attribute requests. Here is a list of all the Attribute requests with the original request and the applicable response.

Note that the request can be initiated by either the Client or Server depending on the operation. For example, a Write is initiated by the GATT Client while a Notification is initiated by the GATT Server.

| BT Spec Chap Ref | Request | Request Data | BT Spec Chap Ref | Response | Response Data |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 3.4.1.1 N/A | Error Response | Request Op Code in Error Attribute Handle in Error Error Code |
| 3.4.2.1 N/A | Exchange MTU | Client Rx MTU | 3.4.2.2 N/A | Exchange MTU response | Server Rx MTU |
| 3.4.3.1 4D.4.8 | Find Information | Starting Handle Ending Handle | 3.4.3.2 4D.4.8 | Find Information Response | Handles Attribute Type UUIDs |
| 3.4.3.3 4D.4.6 | Find by Type Value | Starting Handle Ending Handle Attribute Type Attribute Value | 3.4.3.4 4D.4.64D.4.7 | Find by Type Value Response | Start Handle End of Group Handle |
| 3.4.4.1 4D.4.7 | Read by Type | Starting Handle Ending Handle Attribute Type UUID | 3.4.4.2 4D.4.7 | Read by Type Response | Handle Value Pairs |
| 3.4.4.3 4D.4.1 | Read | Handle | 3.4.4.4 4A | Read Response | Handle, Value |
| 3.4.4.5 N/A | Read Blob | Handle, Offset | 3.4.4.6 N/A | Read Blob Response | Handle, Data |
| 3.4.4.7 N/A | Read Multiple | Handles | 3.4.4.8 N/A | Read Multiple Response | One response with all values concatenated |
| 3.4.4.9 4D.4.5 | Read by Group Type | Starting Handle Ending Handle Attribute Group Type UUID | 3.4.4.10 4D.4.5 | Read by Group Type Response | For each match: Handle, Value Handle of last Attribute in Group |
| 3.4.5.1 4D.4.2 | Write | Handle, Value | There is no Server response to a Write. |  |  |
| 3.4.5.3 4D.4.2 | Write Command | Handle, Value | 3.4.5.2 4A | Write Response | Response code 0x1E Or Error Response |
| 3.4.5.4 N/A | Signed Write Command | Handle, Value, Signature | 3.4.5.2 4A | Write Response | Response code 0x1E Or Error Response |
| 3.4.6.1 N/A | Prepare Write | Handle Offset Value | 3.4.6.2 N/A | Prepare Write Response | Handle Offset Value |
| 3.4.6.3 N/A | Execute Write | Flags (0-cancel, 1-write) | 3.4.6.4 N/A | Execute Write Response | Response code 0x19 Or Error Response |
| 3.4.7.1 4B | Notification | Handle, Value | There is no Client response to a Notification, but you can read about what happens on the Client side in 0 |  |  |
| 3.4.7.2 4B | Indication | Handle, Value | 3.4.7.3 0 | Handle Value Confirmation | Response code 0x1E Or Error Response |

This leads us to some obvious questions: What happens with Read, Write and Notify on the GATT Client side of a connection?  And what about these other operations?

## GATT Client Read

To initiate a read of the value of a Characteristic you need to know two things, the Handle of the Characteristic and the connection ID.   To execute a read you just call:

wiced_bt_util_send_gatt_read_by_handle(conn_id,handle);

This function call will cause the Stack to send a read request to the GATT Server.  After some time, you will get a callback in your GATT event handler with the event code GATT_OPERATION_CPLT_EVENT.  The callback parameter can then be cast into a wiced_bt_gatt_operation_complete_t.   This structure has everything you need.  Specifically:

/** Response to read/write/disc/config operations (used by GATT_OPERATION_CPLT_EVT notification) */

typedef struct

{

uint16_t                conn_id;            /**< ID of the connection */

wiced_bt_gatt_optype_t  op;                 /**< Type of operation completed */

wiced_bt_gatt_status_t  status;             /**< Status of operation */

wiced_bt_gatt_operation_complete_rsp_t  response_data;  /**< Response data */

} wiced_bt_gatt_operation_complete_t;

This same event is used to handle many of the responses from a GATT Server.  Exactly which response can be determined by the wiced_bt_gatt_optype_t which is just an enumeration of operations.  For instance, 0x02 means you are getting the response from a read.

enum wiced_bt_gatt_optype_e

{

GATTC_OPTYPE_NONE             = 0,    /**< None      */

GATTC_OPTYPE_DISCOVERY        = 1,    /**< Discovery */

GATTC_OPTYPE_READ             = 2,    /**< Read      */

GATTC_OPTYPE_WRITE            = 3,    /**< Write     */

GATTC_OPTYPE_EXE_WRITE        = 4,    /**< Execute Write */

GATTC_OPTYPE_CONFIG           = 5,    /**< Configure */

GATTC_OPTYPE_NOTIFICATION     = 6,    /**< Notification */

GATTC_OPTYPE_INDICATION       = 7     /**< Indication */

};

The wiced_bt_gatt_status_t is an enumeration different error codes from the enumeration wiced_bt_gatt_status_e which was introduced in Chapter 4A. As a reminder, the first few values are:

enum wiced_bt_gatt_status_e

{

WICED_BT_GATT_SUCCESS                    = 0x00,         /**< Success */

WICED_BT_GATT_INVALID_HANDLE             = 0x01,         /**< Invalid Handle */

WICED_BT_GATT_READ_NOT_PERMIT            = 0x02,         /**< Read Not Permitted */

WICED_BT_GATT_WRITE_NOT_PERMIT           = 0x03,         /**< Write Not permitted */

WICED_BT_GATT_INVALID_PDU                = 0x04,         /**< Invalid PDU */

The last piece is the response data, wiced_bt_gatt_operation_complete_rsp_t which contains the handle and an attribute value structure which contains the actual GATT data along with information about it.

typedef union

{

wiced_bt_gatt_data_t    att_value;    /**< Response data for read operations

(initiated using #wiced_bt_gatt_send_read) */

uint16_t                mtu;          /**< Response data for configuration

operations */

uint16_t                handle;       /**< Response data for write operations

(initiated using #wiced_bt_gatt_send_write) */

} wiced_bt_gatt_operation_complete_rsp_t; /**< GATT operation complete response type */

The attribute value structure containing the GATT data looks like this:

/**  Response data for read operations */

typedef struct

{

uint16_t                    handle;     /**< handle */

uint16_t                    len;        /**< length of response data */

uint16_t                    offset;     /**< offset */

uint8_t                     *p_data;    /**< attribute data */

} wiced_bt_gatt_data_t;

These structures look a bit overwhelming, but you can easily use this information to find out what happened.  Here is an example of how you might deal with this callback to print out the response and all the raw bytes of the data that were sent.

case GATT_OPERATION_CPLT_EVT:

// When you get something back from the peripheral... print it out.. the data

WICED_BT_TRACE("Event Complete Conn=%d Op=%d status=0x%X Handle=0x%X len=%d Data=",

p_data->operation_complete.conn_id,

p_data->operation_complete.op,

p_data->operation_complete.status,

p_data->operation_complete.response_data.handle,

p_data->operation_complete.response_data.att_value.len);

for(int i=0;i<p_data->operation_complete.response_data.att_value.len;i++)

{

WICED_BT_TRACE("%02X ",

p_data->operation_complete.response_data.att_value.p_data[i]);

}

WICED_BT_TRACE("\r\n");

break;

## GATT Client Write and Write Command

In order to send a GATT Write all you need to do is make a structure of type wiced_bt_gatt_value_t, setup the handle, offset, length, authorization and value then call wiced_bt_gatt_send_write.  Here is an example:

wiced_bt_gatt_value_t *p_write = ( wiced_bt_gatt_value_t* )wiced_bt_get_buffer( sizeof( wiced_bt_gatt_value_t ) + sizeof(myData)-1);

if ( p_write )

{

p_write->handle   = myHandle;

p_write->offset   = 0;

p_write->len      = sizeof(myData);

p_write->auth_req = GATT_AUTH_REQ_NONE;

memcpy(p_write->value,myData,sizeof(myData));

wiced_bt_gatt_send_write ( conn_id, GATT_WRITE, p_write );

wiced_bt_free_buffer( p_write );

The second argument to wiced_bt_gatt_send_write function can either be GATT_WRITE or GATT_WRITE_NO_RSP depending on whether or not you want a response from the server.

The one trick is that the length of the structure is variable with the length of the data.  In the above example, you allocate a block big enough to hold the structure + the length of the data minus 1.  Then use a pointer to fill in the data.

IF you asked for a write with response (i.e. GATT_WRITE), sometime after you call the wiced_bt_gatt_send_write you will get a GATT callback with the event code GATT_OPERATION_CPLT_EVT.  You can then figure out if the write was successful by checking to see if you got a Write Response or an Error Response (with the error code from the wiced_bt_gatt_status_e enumeration).

Don’t forget the you can only issue one Read/Write at a time and that you cannot send the next Read/Write until the last one is finished.

## GATT Client Notify and Indicate

When the GATT Server initiates a Notify or an Indicate you will get a GATT callback with the event code set as GATT_OPERATION_CPLT_EVT.  You will see that the Operation value is GATTC_OPTYPE_NOTIFICATION or GATTC_OPTYPE_INDICATE and the value is just like the Read Response.  In the case of Indicate you can return WICED_BT_GATT_SUCCESS which means the Stack will return a Handle Value Confirmation to the Client, or you can return something other than WICED_BT_GATT_SUCCESS in which case the Stack sends an Error Response with the code that you choose from the wiced_bt_gatt_status_e enumeration. Remember from Chapter 4A that the (partial) list of available return values is:

enum wiced_bt_gatt_status_e

{

WICED_BT_GATT_SUCCESS                 = 0x00,     /**< Success */

WICED_BT_GATT_INVALID_HANDLE          = 0x01,     /**< Invalid Handle */

WICED_BT_GATT_READ_NOT_PERMIT         = 0x02,     /**< Read Not Permitted */

WICED_BT_GATT_WRITE_NOT_PERMIT        = 0x03,     /**< Write Not permitted */

WICED_BT_GATT_INVALID_PDU             = 0x04,     /**< Invalid PDU */

WICED_BT_GATT_INSUF_AUTHENTICATION    = 0x05,     /**< Insufficient Authentication */

WICED_BT_GATT_REQ_NOT_SUPPORTED       = 0x06,     /**< Request Not Supported */

WICED_BT_GATT_INVALID_OFFSET          = 0x07,     /**< Invalid Offset */

WICED_BT_GATT_INSUF_AUTHORIZATION     = 0x08,     /**< Insufficient Authorization */

WICED_BT_GATT_PREPARE_Q_FULL          = 0x09,     /**< Prepare Queue Full */

WICED_BT_GATT_NOT_FOUND               = 0x0a,     /**< Not Found */

WICED_BT_GATT_NOT_LONG                = 0x0b,     /**< Not Long Size */

WICED_BT_GATT_INSUF_KEY_SIZE          = 0x0c,     /**< Insufficient Key Size */

WICED_BT_GATT_INVALID_ATTR_LEN        = 0x0d,     /**< Invalid Attribute Length */

WICED_BT_GATT_ERR_UNLIKELY            = 0x0e,     /**< Error Unlikely */

WICED_BT_GATT_INSUF_ENCRYPTION        = 0x0f,     /**< Insufficient Encryption */

WICED_BT_GATT_UNSUPPORT_GRP_TYPE      = 0x10,     /**< Unsupported Group Type */

WICED_BT_GATT_INSUF_RESOURCE          = 0x11,     /**< Insufficient Resource */

## GATT Group

There is one last GATT concept that needs to be introduced to understand the next GATT Procedures. That is the Group.  A Group is a range of handles starting at a Service, Service Include or a Characteristic and ending at the last Handle that is associated with the Group.  To put it another way, a Group is all of the rows in the GATT database that logically belong together. For instance, in the GATT database below, the Service Group for <<Generic Access>> starts at Handle 0x14 and ends at 0x18.

## GATT Client Read by Group Type

The GATT Client Read by Group Type request takes as input a search starting Handle, search ending Handle and Group type. It outputs a list of tuples with the Group start Handle, Group end Handle, and value.  This request can only be used for a “Grouping Type” meaning, <<Service>>, <<Included Service>> and <<Characteristic>>.

Consider this GATT Database from Chapter 4B Exercise 3 (pairing) and Chapter 4C Exercise 1 (advertise manufacturer's data).

| Handle | Type | Value |
| --- | --- | --- |
| 0x01 | <<Service>> | <<Generic Attribute>> |
| 0x14 | <<Service>> | <<Generic Access>> |
| 0x15 | <<Characteristic>> | 0x16, <<Device Name>> |
| 0x16 | <<Device Name>> |  |
| 0x17 | <<Characteristic>> | 0x18, <<Appearance>>,0x02 |
| 0x18 | << Appearance>> |  |
| 0x28 | <<Service>> | __UUID_WICED101 |
| 0x29 | <<Characteristic>> | 0x2A, __UUID_WICED101_LED,0x0E |
| 0x2A | __UUID_WICED101_LED |  |
| 0x2B | <<Characteristic>> | 0x2C, __UUID_WICED101_BUTTONS,0x12 |
| 0x2C | __UUID_WICED101_BUTTONS |  |
| 0x2D | <<CCCD>> |  |

In the database above if you input search start Handle=0x01, search end Handle=0xFFFF and Group Type = <<Service>> you would get as output:

| Group Start Handle | Group End Handle | UUID |
| --- | --- | --- |
| 0x01 | 0x01 | <<Generic Attribute>> |
| 0x14 | 0x18 | <<Generic Access>> |
| 0x28 | 0x2D | __UUID_WICED101 |

In words, you receive a list of all the Service UUIDs with the start Handle and end Handle of each Group.

## GATT Client Find by Type Value

The GATT Client "Find by Type Value" request takes as input the search starting Handle, search ending Handle, Attribute Type and Attribute Value.  It then searches the Attribute database and returns the starting and ending Handles of the Group that match that Attribute Type and Attribute Value.  This function was put into GATT specifically to find the range of Handles for a specific Service.

Consider the example above.  If your input to the "Find by Type Value" was starting handle=0x01, ending handle=0xFFFF, Type=<<Service>> and Value=__UUID_WICED101 the output would be:

| Group Start Handle | Group End Handle |
| --- | --- |
| 0x28 | 0x2D |

This function cannot be used to search for a specific Characteristic because the Attribute value of a Characteristic declaration cannot be known a-priori. This is because the Characteristic Properties and Characteristic Value Attribute Handle (which are part of the Attribute Value) are not known up front. As a reminder, here is the Characteristic declaration:

## GATT Client Read by Type

The GATT Client "Read by Type" request takes as input the search starting Handle, search ending Handle and Attribute Type. It outputs a list of Handle value pairs.  In the example above if you entered the search starting Handle=0x28, search ending Handle=0x2D and Type=<<Characteristic>> you would get as output:

| Characteristic Handle | Value Handle | UUID | GATT Permission |
| --- | --- | --- | --- |
| 0x29 | 0x2A | __UUID_WICED101_LED | 0x0E |
| 0x2B | 0x2C | __UUID_WICED101_BUTTONS | 0x12 |

In words, you get back a list of the Characteristic Handles, the Handles of the Values, the UUIDs, and the GATT Permissions.

## GATT Client Find Information

The input to the GATT Client "Find Information Request" is simply a search starting Handle and a search ending Handle.  The GATT Server then responds with a list of every Handle in that range, and the Attribute type of the handle.  Notice that this is the only GATT procedure that returns the Attribute Type.

If you execute a GATT Client Find Information with the handle range set to 0x180x27 you will get a response of:

| Handle | Attribute Type |
| --- | --- |
| 0x18 | <<Appearance>> |

In words, the attribute type that is associated with the Characteristic handle 0x18.

# GATT Procedure: Service Discovery

Given that all transactions between the GATT Client and GATT Server use the “Handle” instead of the UUID, one huge question left unanswered is how do you find the Handles for the different Services, Characteristics and Descriptors on the GATT Server?  A very, very bad answer to that question is that you hardcode the handles into the GATT Client.  A much better answer is that you do Service Discovery.  The phrase Service Discovery includes discovering all the Attributes of a device including Services, Characteristics and Descriptors.  This is done using the GATT Procedures that were introduced in sections just above: Read by Group Type, Find by Type Value, Read by Type and Find Information.

## Service Discovery Algorithm

The steps in the Service discovery algorithm are:

- Discover all the Services (which gives you the UUID and Start and End Handles of all the Service Groups) using Read by Group Type (to discover all Services).  Or possibly discover only one Service by using Find by Type Value.

- For each Service Group discover all the Characteristics (which gives you the Handles and UUIDs) using Read by Type with the Handle range of the Service Group that you discovered in step (1).

- Using the Characteristic Handles from (2) you can then calculate the start and end Handle ranges of each of the Descriptors for each Characteristic, then run a Read by Type with the Type set to <<Characteristic>>.

- After (3) is done you will know the Handle of all the Characteristics in a service.  You will then need to calculate the ranges of the Descriptors for the Characteristics.

- Using the ranges from (4) discover the Descriptors using the GATT Procedure Find Information.

## WICED Service Discovery

Inside of WICED we have provided some utility functions to simplify the Service Discovery process.  To include these functions in your project you need to add the GATT utilities library to your makefile.

$(NAME)_COMPONENTS += gatt_utils_lib.a

The library has one Service discovery API that can discover Services, Characteristics and Descriptors.

wiced_bt_gatt_status_t wiced_bt_gatt_send_discover (uint16_t conn_id,

wiced_bt_gatt_discovery_type_t discovery_type,

wiced_bt_gatt_discovery_param_t *p_discovery_param );

The discovery type is an enumeration.  (GATT_DISCOVER_MAX is not a legal parameter)

enum wiced_bt_gatt_discovery_type_e

{

GATT_DISCOVER_SERVICES_ALL = 1,   			/**< discover all services */

GATT_DISCOVER_SERVICES_BY_UUID,   		/**< discover service by UUID */

GATT_DISCOVER_INCLUDED_SERVICES,  		/**< discover an included service within a service */

GATT_DISCOVER_CHARACTERISTICS,   		 	/**< discover characteristics of a service*/

GATT_DISCOVER_CHARACTERISTIC_DESCRIPTORS, 	/**< discover characteristic descriptors */

GATT_DISCOVER_MAX                           			/* maximum discovery types */

};

The discovery parameter contains:

typedef struct

{

wiced_bt_uuid_t uuid;        	/**< Service or Characteristic UUID */

uint16_t        s_handle;    	/**< Start handle for range to search */

uint16_t        e_handle;    	/**< End handle for range to search */

} wiced_bt_gatt_discovery_param_t;

After you call this function, the stack will issue the correct GATT Procedure.  Then, each time the GATT Server responds with some information you will get a GATT callback with the event type set to GATT_DISCOVERY_RESULT_EVT.  The event parameter can then be decoded using a set of macros provided by WICED.

/*************************************************************************

*  Macros for parsing results GATT discovery results

* (while handling GATT_DISCOVERY_RESULT_EVT)

****************************************************************************/

/* Discovery type: GATT_DISCOVER_SERVICES_ALL or GATT_DISCOVER_SERVICES_BY_UUID */

#define GATT_DISCOVERY_RESULT_SERVICE_START_HANDLE(p_event_data)

#define GATT_DISCOVERY_RESULT_SERVICE_END_HANDLE(p_event_data)

#define GATT_DISCOVERY_RESULT_SERVICE_UUID_LEN(p_event_data)

#define GATT_DISCOVERY_RESULT_SERVICE_UUID16(p_event_data)

#define GATT_DISCOVERY_RESULT_SERVICE_UUID32(p_event_data)

#define GATT_DISCOVERY_RESULT_SERVICE_UUID128(p_event_data)

/* Discovery type: GATT_DISCOVER_CHARACTERISTIC_DESCRIPTORS */

#define GATT_DISCOVERY_RESULT_CHARACTERISTIC_DESCRIPTOR_UUID_LEN(p_event_data)

#define GATT_DISCOVERY_RESULT_CHARACTERISTIC_DESCRIPTOR_UUID16(p_event_data)

#define GATT_DISCOVERY_RESULT_CHARACTERISTIC_DESCRIPTOR_UUID32(p_event_data)

#define GATT_DISCOVERY_RESULT_CHARACTERISTIC_DESCRIPTOR_UUID128(p_event_data)

#define GATT_DISCOVERY_RESULT_CHARACTERISTIC_DESCRIPTOR_VALUE_HANDLE(p_event_data)

/* Discovery type: GATT_DISCOVER_CHARACTERISTICS */

#define GATT_DISCOVERY_RESULT_CHARACTERISTIC_VALUE_HANDLE(p_event_data)

#define GATT_DISCOVERY_RESULT_CHARACTERISTIC_UUID_LEN(p_event_data)

#define GATT_DISCOVERY_RESULT_CHARACTERISTIC_UUID16(p_event_data)

#define GATT_DISCOVERY_RESULT_CHARACTERISTIC_UUID32(p_event_data)

#define GATT_DISCOVERY_RESULT_CHARACTERISTIC_UUID128(p_event_data)

There really should be a macro called GATT_DISCOVERY_RESULT_CHARACTERISTIC_HANDLE but for some reason, it isn't in WICED (yet). Therefore, when you want to get a Characteristic's Handle during Characteristic Discovery in your callback, you will have to use something like this:

p_data->discovery_result.discovery_data.characteristic_declaration.handle

When the discovery is complete you will get one more GATT callback with the event type set to GATT_DISCOVERY_CPLT_EVT.

Your firmware would typically be a state machine that would sequence through the Service, Characteristic and Descriptor discoveries as the GATT_DISCOVERY_CPLT_EVT is completed.

# Running a GATT Server

Although somewhat uncommon, there is no reason why a BLE Central cannot run a GATT Server.  In other words, all combinations of GAP Peripheral/Central and GATT Server/Client are legal.  An example of this might be a TV that has a BLE remote control.  Recall that the device that needs to save power is always the Peripheral, in this case the Remote Control.  However, the TV is the thing being controlled, so it would have the GATT database remembering things like channel, volume, etc.

The firmware that you write on a Central to run a GATT database is EXACTLY the same as on a Peripheral.

# Exercises

To do this exercise will require two development kits.  One to be your Peripheral, which should be programmed with the firmware from chapter 04B exercise xx or 04C exercise YY.

Make an Observer

This project will listen to all the BLE devices that are broadcasting.  It should print out the BD Address of each device.  To build this project follow these steps:

Run BT designer and make a project with no GATT database

Delete the advertising functions and function calls

Make a function to process the scanned advertising packets which just prints out the BD Address of the remote.  This function declaration should look like this:

void myScanCallback (wiced_bt_ble_scan_results_t *p_scan_result, uint8_t *p_adv_data);

Add a call to wiced_bt_ble_scan in the BTM_ENABLED event with a function pointer to your advertiser processor function.

Build and Program

What is the cause of “Unhandled Bluetooth Management Event: 0x16 (22)” and how do you fix it?

Add a filter to show only your device

Copy Exercise 01.  Change your advertising packet call back to only print out devices that match

- Your Device Name

- Your Service UUID

- Your Manufacturer's Code

You can use the function wiced_bt_ble_check_advertising_data to look at the advertising packet and find fields of type wiced_bt_ble_advert_type_t.  If there is a field that matches it will return a pointer to those bytes and a length.

You can use strncmp (to compare strings) and memcmp (to compare UUIDs) to see if the fields match what you are looking for.

Update to connect to your Peripheral device & turn on/off the LED

Your project will scan and find your peripheral.  Then connect to it.  Then let you send 0’s and 1’s to led characteristic.  To simplify the project, you will hardcode the handle, but in the upcoming exercises you will add service discovery.

The steps are as follows:

Copy the previous project

Build and program it to make sure it still works

Add a keyboard interface that has a ‘?’ To print out help.  Remember from chapter 2, you will need to initialize the puart and setup a call back in the _app_init function.

Create an RxCallback function to handle key presses.  Make a switch with one case per key (used by this program).

Add keys to turn on/off scanning ‘s’ ON and ‘S’ off (call the wiced_bt_ble_scan with the correct arguments)

Remove the start scan from the BTM_Enabled Event

Program your projects and make sure that the keyboard interface works and that the s/S turns the scanning on and off.

Update your adv packet function to connect to the Peripheral when it finds one that it recognizes by calling wiced_bt_gatt_le_connect.

After starting the connection, turn off scanning

Create a new function to serve as the gatt callback function.  The easiest way to do this is to copy the callback from one of the previous projects and then remove all the cases except the default case.  Regardless, the function must match this prototype:

typedef wiced_bt_gatt_status_t wiced_bt_gatt_cback_t(wiced_bt_gatt_evt_t event, wiced_bt_gatt_event_data_t *p_event_data);

Register your GATT callback function in the BTM enabled event with function wiced_bt_gatt_register

Add a global variable uint16_t to save the connection id.

In the gatt callback when you get a GATT_CONNECTION_STATUS_EVT figure out if it is a connect or a disconnect.  Then save the connection id to your global variable

Add a ‘d’ to call disconnect

Program your project.  Make sure that it finds and connects to your device.  Then disconnects.  Then start the scanning and make sure that it reconnects when you press ‘s’

Notice the Unhandled BTM event 0x16 message.  Add a case to print out the “right” message

Create a new global variable called ledHandle.  HARDCODE its initial value to the handle of your LED Characteristic.  You won’t change the variable in this exercise, but in a future one you will find the handle via a service discovery.

Create a new function to write the led with a uint8_t argument that represents either 1 or 0.  If there is no connection or the ledHandle is 0 then you should return.  Then call the GATT write function.

Add cases for ‘1’ and ‘0’ in your command that call your write LED function with 0 or 1

Program and test

(Advanced) Add keys to turn on CCCD

In this project, we will add setting up the CCCD to turn on notifications for the button Characteristic, and printing out messages when the notifications happens.

Copy the previous project

Build and program it to make sure that it still works.

Add a new global variable called cccdHandle to hold the handle of the CCCD.  Setup its initial value to 0x2D (this is a hardcode which we will fix in the next exercise).  Depending on how you set the GATT database your handle might be different.

Add cases ‘n’ and ‘N’ to set and unset the CCCD using the function called wiced_bt_util_set_gatt_client_config_descriptor which does all the work of setting up a CCCD write.  You should probably check to make sure that you have a valid connection ID and CCCD Handle before you call the write function.

Add those keys to the help print out message.

Add a case for GATT_OPERATION_CPLT_EVT into the GATT callback.  This case should print out the connection id, operation, status, handle, length and the raw bytes of data

Build Program and Test

(Advanced) Make your project do Service Discovery

Instead of hardcoding the Handle for the LED and Button CCCD, we will modify our program to do a Service discovery.   Instead of triggering the whole process with a state machine, we will use keyboard commands to launch the three stages.

The three stages are:

- ‘q’= Service discovery with the UUID of the WICED101 Service

- ‘w’=Characteristic discovery with the range of handles from (1)

- ‘e’ =Descriptor discovery of the button Characteristic to find the CCCD.

Copy your previous project.

Create variables to save the start and end Handle of the WICED101 Service.  And create a variable to hold the WICED101 Service UUID

static const uint8_t serviceUUID[] = {0xF0 ,0x34 ,0x9B ,0x5F ,0x80 ,0x00 ,0x00 ,0x80 ,0x00 ,0x10 ,0x00 ,0x00 ,0x00 ,0x00 ,0x00 ,0x00 };

static uint16_t serviceStartHandle=1;

static uint16_t serviceEndHandle=0xFFFF;

Make a new structure to manage the discovered handles of Characteristics.

typedef struct {

uint16_t startHandle;

uint16_t endHandle;

uint16_t valHandle;

uint16_t cccdHandle;

} charHandle_t;

Create a charHandle_t for the LED and the Button as well as the Characterstic UUIDs to search for.

static const uint8_t ledUUID[] = {0xF1u,0x34u,0x9Bu,0x5Fu,0x80u,0x00u,0x00u,0x80u,0x00u,0x10u,0x00u,0x00u,0x00u,0x00u,0x00u,0x00u};

static charHandle_t ledChar;

static const uint8_t buttonUUID[] = {0xF2u,0x34u,0x9Bu,0x5Fu,0x80u,0x00u,0x00u,0x80u,0x00u,0x10u,0x00u,0x00u,0x00u,0x00u,0x00u,0x00u };

static charHandle_t buttonChar;

Create an array of charHandle_t to temporarily hold the Characteristics.  When you discover the Characteristics, you won’t know what order will they occur so you need to save the Handles to calculate the end of group Handles.

#define MAX_CHARS_DISCOVERED (10)

static charHandle_t charHandles[MAX_CHARS_DISCOVERED];

static uint32_t charHandleCount;

Add a function to launch the service discovery called “startServiceDiscovery”.  This function will be called when the user presses ‘q’.  Instead of finding all the UUIDs you will turn on the filter for just the WICED101 UUID.  Setup the wiced_bt_gatt_discovery_param_t with the starting and ending handle set to 0x01 & 0xFFFF.  Setup the UUID to be the UUID of the WICED101 Service.  Use memcpy to copy the Service UUID into the wiced_bt_gatt_discovery_param_t;  Set the discovery type to GATT_DISCOVER_SERVICES_BY_UUID and launch the wiced_bt_gatt_send_discover.

Add the case GATT_DISCOVERY_RESULT_EVT to your GATT Event Handler.  If the discovery type is GATT_DISCOVER_SERVICES_BY_UUID then update the serviceStart and serviceEnd handle with the actual start and end Handles (remember GATT_DISCOVERY_RESULT_SERVICE_START_HANDLE and GATT_DISCOVERY_RESULT_SERVICE_END_HANDLE

Add a function to launch the Characteristic called “startCharacteristicDiscovery” when the user presses ‘w’.  Setup the wiced_bt_gatt_discovery_param_t start and end Handle to be the range you discovered in the previous step.  Then call wiced_bt_gatt_send_discover with the discovery type set to GATT_DISCOVER_CHARACTERISTICS.

In the GATT_DISCOVERY_RESULT_EVT of your GATT Event Handler add an if case for the characteristic result.  In the “if” you need to save the start, valuehandle in your charHandles array.  Set the end handle to the end of the Service handle (assume that this Group ends with the Service).  If this is not the first Characteristic then set the previous Characteristic end handle

if(charHandleCount != 0)

{

charHandles[charHandleCount-1].endHandle = charHandles[charHandleCount].endHandle-1;

}

charHandleCount += 1;

The point is to assume that the characteristic ends at the end of the service handle.  But, if you find another Characteristic then you know that the end of the previous Characteristic is this one.

Then you want to see if the Characteristic is the Button or LED Characteristic,  if so then also save the start, end and value handles.

Add a function to launch the Descriptor discovery called “startDescriptorDiscovery” when the user presses ‘e’.  The purpose of this function is to find the CCCD Handle for the button Characteristic.  It will need to search for all the Descriptors in the Characteristic Group.  The start will be the Button Value Handle + 1 to the end of the group handle.  Once the parameters are setup, launch wiced_bt_gatt_send_discover with GATT_DISCOVER_CHARACTERISTIC_DESCRIPTORS

In the GATT_DISCOVERY_RESULT_EVT of your GATT Event Handler add an if case for the Descriptor result.  If the Discriptor Attribute is <<CCCD> == 0x2902 then save the Button CCCD Handle.

The last change you need to make is for the GATT_DISCOVERY_CPLT_EVT.  You should check to see if it is a Characteristic discovery.  If it is, then you will be able figure out the endHandle for each of the LED and Button Characteristic by copying them from the charHandles array.  Your code could look something like this:

// Once all characteristics are discovered... you need to setup the end handles

if(p_data->discovery_complete.disc_type == GATT_DISCOVER_CHARACTERISTICS)

{

for(int i=0;i<charHandleCount;i++)

{

if(charHandles[i].startHandle == ledChar.startHandle)

ledChar.endHandle = charHandles[i].endHandle;

if(charHandles[i].startHandle == buttonChar.startHandle)

buttonChar.endHandle = charHandles[i].endHandle;

}

}

Add the function calls for ‘q’,’w’, and ‘e’.  Also add those buttons to the help print out.  Then run and test your project.

‘s’ Start Scanning

‘q’ Discover the WICED101 Service

‘w’ Discover the Button and LED Characteristics

‘e’ Discover the Button CCCD

‘1’ Turn on the LED

‘0’ Turn off the LED

‘n’ Turn on notifications (then press the button on the Peripheral and make sure it works)

‘N’ Turn off the notification and make sure they are off

(Advanced) Run the advertising scanner